4A15 Hydraulics · Dimensional analysis

Tutorial: a weir model study

A broad-crested weir is to be studied with Froude-scaled physical models at 1:2 and 1:4. The prototype, drawn to scale in the hydraulician app (exercise DA-3), has crest height P = 0.696 m above the approach bed and crest length Lc = 1.739 m, and passes qp = 0.78 m²/s per metre width. Answer Questions 1–3 before the session; Question 4 is done in the app; 5 and 6 afterwards.

1. Dimensional analysis

The discharge per metre width q over the weir depends on the head H, the crest height P, the crest length Lc, gravity g, and the water's density ρ, viscosity μ and surface tension σ.

(a) How many Π groups describe the problem? Using ρ, g and H as repeating variables, find them.

(b) Hence show that q = Cd √g H3/2 with Cd = φ(H/P, Lc/H, ReH, WeH), where ReH = ρ√g H3/2/μ and WeH = ρgH²/σ. Which force ratio does each of ReH and WeH compare?

2. Designing the models

(a) Which dimensionless group must be the same in model and prototype for gravity-dominated flow over a weir, and why can ReH and WeH not be matched at the same time with water in both?

(b) For λ = ½ and λ = ¼ find the model crest height, crest length and the discharge per metre width that must be supplied. (In this vertical slice q is per metre, so it scales as λ3/2, not λ5/2.)

(c) The prototype takes about 55 s to settle to steady flow. How long should each model take?

3. Predictions

(a) For an ideal broad-crested weir (critical depth over a long crest, no losses, approach velocity neglected) q = (2/3)3/2 √g H3/2. Predict the prototype head from qp.

(b) If the prototype head turns out to be Hp, what heads do you predict for the two models, and what Cd? Write the relationship; you will fill in the numbers in the session.

4. In the app (exercise DA-3)

For each rung (the box on the card: 0 = prototype, 1 = 1:2, 2 = 1:4) set Inflow q to your value from 2(b) (0.780 m²/s for the prototype) and the Reservoir level printed on the card, press R, wait for the settle, then read the steady approach-pool depth d on a gauge at the card's station. Compute H = d − P and Cd, and complete the table.

|  | prototype λ = 1 | 1:2 model | 1:4 model |
| --- | --- | --- | --- |
| q (m²/s) | 0.7800 | 0.2758 | 0.0975 |
| P (m) | 0.696 | 0.348 | 0.174 |
| d measured (m) |  |  |  |
| H = d − P (m) |  |  |  |
| H predicted = λHp (m) |  |  |  |
| Cd |  |  |  |

5. Scale effects

(a) Evaluate ReH and WeH for the prototype and both models, with ν = 1.0×10⁻⁶ m²/s and σ = 0.073 N/m. Would a real 1:4 model of this weir, in water, suffer from viscous or surface-tension scale effects?

(b) A lower limit often used for model weir heads is a few centimetres; take 30 mm. What is the smallest scale at which this weir could be modelled?

6. The simulation's own scale effect

The app has no real viscosity or surface tension at these scales; its equivalent is the grid. Its cells are Δx = 21.7 mm at Medium and do not shrink with the model. How many cells span H on each rung? If H needs at least about 8 cells to be resolved, what is the smallest model this simulation can represent at Medium? What would you expect to happen to Cd at λ = ¼ on a coarser grid?