4A15 Hydraulics · Dimensional analysis

Tutorial: dimensional analysis and a sloshing tank

A design chart is wanted for the natural sloshing period T of rectangular tanks of length B holding water to depth d. An analyst plans to compute T by creating a set of models, with five values each of B, d and the initial tilt a of the surface (below). Viscosity and surface tension are negligible.

1. Dimensional analysis to guide model building

(a) How many models does the analyst plan?

(b) T depends on B, d, a, g and the density ρ. Find the Π groups.

(c) For small tilts the period does not depend on a. Show that T can then be written as

T=Kdg, with K=ϕ(B/d),

and say how many models the design chart needs.

(d) In a long, shallow tank the surface disturbance travels at gd and goes the length of the tank and back in one period. Find K.

(e) Deep water is barely disturbed by a wave, so in a deep tank T does not depend on d. Use dimensional analysis to find an expression for T in a deep tank, and write it in terms of K from (c).

(f) Linear wave theory gives

T=2π/gk tanh(kd), with k=π/B.

Show that

Ktheory=2π/kd tanh(kd)

depends on Bd only, and that it reduces to (d) and (e).

2. In the session (exercise DA-3)

A model gives a period T in seconds for one particular tank. Dividing by dg removes the tank's size and leaves

Kapp=Tgd

a pure number that can be compared directly with any other tank's, and with Ktheory. By 1(c), one model per value of Bd is enough to draw the analyst's design chart: the five models below stand in for all 125.

Place a Depth gauge at x=0.5 m, low in the water. For each model set Tank length and Water depth in Controls → Geometry, with Initial tilt a/d=0.2. Expand the gauge (⤢) and read the times t1 and t4 of the first and fourth crests after release; T=(t4−t1)/3.

| Model | B (m) | d (m) | Bd | t1 (s) | t4 (s) | T (s) | Kapp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | 1 | 1 | 1 |  |  |  |  |
| B | 2 | 1 | 2 |  |  |  |  |
| C | 4 | 1 | 4 |  |  |  |  |
| D | 8 | 1 | 8 |  |  |  |  |
| E | 8 | 0.5 | 16 |  |  |  |  |

Plot Kapp against Bd on the chart, which shows Ktheory and the two limits from 1(d) and 1(e).

(a) Do your five points lie on one curve? How well does Ktheory describe it?

(b) Without building a model, use the chart to find the period of a tank 20 m long holding 2.5 m of water.

Answers

1(a)  125

1(b)  Tgd, Bd, ad

1(c)  5 models

1(d)  K=2B/d

1(e)  T=CBg, so K=CBd

1(f)  shallow: K→2B/d; deep: K→2πBd, so C=2π

2  Kapp≈ 3.8, 5.4, 8.9, 17, 33 for models A–E

2(a)  yes, within a few per cent of Ktheory

2(b)  Ktheory=16.4, so T=8.3 s