Hydropower and unsteady flow — tutorial sheet

Complete by hand before the unsteady flow tutorial. The scheme is hydraulician exercise HP-3, so every answer can be checked on screen.

# The scheme

Figure 1: The scheme and its notation, to scale. Heights are metres above the datum z = 0.

Figure 1 shows a hydropower scheme. The reservoir surface stands at 24.9 m. A level headrace, L = 42.4 m long and Dh = 3.05 m deep, runs to a surge shaft Ds = 3.0 m wide, open to the atmosphere; a penstock (Dp = 2.4 m, Lp ≈ 20 m) drops to a nozzle 0.48 m wide at z = 3.0 m, discharging to atmosphere. The valve at the nozzle is the turbine, and closes instantaneously on shutdown. The scheme is a slice one metre wide, so discharges are per metre of width (m2/s) and a conduit's area is its depth: A = Dh and As = Ds.

Take q0 = 7.6 m2/s, a Darcy friction factor λ = 0.03 in both conduits, g = 9.81 m/s2 and ρ = 1000 kg/m3, and neglect local losses. The conduits are slots of depth D and unit width, not circular pipes: the hydraulic radius is D/2, so the hydraulic diameter 4Rh = 2D takes the place of the pipe diameter in Darcy–Weisbach,

hf = λ · (L/2D) · V2/2g

## Notation

V = q/D is the mean velocity in a conduit; H = 24.9 − 3.0 = 21.9 m the gross head, reservoir surface to nozzle axis; hf the friction loss. η is the shaft water level relative to the reservoir surface, positive above it: η0 (negative) is the steady running level and ηmax the first crest. k is the loss coefficient in η0 = −k·V02, and Y = L·A/(2·g·k·As).

# Part 1: Hydropower — penstock

Energy from the reservoir surface to the jet gives Vjet = √(2g·(H − hf)), hf being the friction in headrace and penstock together, and the power delivered to the nozzle is P = ρ·g·q·(H − hf). Since V ∝ q in both conduits, hf ∝ q2.

Find:

- the headrace velocity, V0;

[Answer: 2.49 m/s]

- the friction loss along the headrace, hf;

[Answer: 0.066 m]

- the jet speed, Vjet;

[Answer: Vjet = 20.7 m/s]

- the power P delivered to the nozzle, per metre of width;

[Answer: 1.62 MW/m]

- the discharge q* at which P would be greatest.

[Answer: 57 m2/s]

# Part 2: Unsteady flow — surge tower

The shaft carries no flow and is open to the air, so its water stands at the piezometric head at the knee: the velocity head V02/2g plus the friction loss hf below the reservoir surface. Write this as η0 = −k·V02. (The lectures take k as friction alone, which is fine for a tunnel a thousand bores long; this headrace is 14 bores long and the velocity head is most of the drawdown.)

Find:

- the steady level in the shaft, η0;

[Answer: −0.38 m]

- the loss coefficient k.

[Answer: 0.0616 s2/m]

After instantaneous shutdown the lecture's V2–η solution gives the first crest, where V = 0, from

ηmax = Y · [1 − exp((η0 − ηmax)/Y)]

Find:

- Y;

[Answer: Y = 35.7 m]

- ηmax, by trial and error.

[Hint: bracket the zero of F(η) = η − Y·[1 − exp((η0 − η)/Y)], as in the lecture example. Answer: ηmax = 4.97 m]

# Part 3 — In the session

Before the session, fill the predicted column from Parts 1 and 2. In the session: hover the headrace mid-length for V0 and the jet just past the nozzle for Vjet; set the gauges to Level η and read the reservoir by the wall and the shaft: η0 = shaft − reservoir; then switch to d, press V, and read the first crest's rise above the pre-slam level: ηmax = rise + η0.

Read η0 on η, not on h: the shaft is open to the air, so its level is the reading you want, and p/ρg part-way down a slowly circulating column is both noisier and biased. The reservoir slider says 25.0 m while a gauge by the wall reads the 24.9 m the calculation uses. Let the settle finish, and press R before each fresh slam.

| quantity | predicted | measured |
| --- | --- | --- |
| V0 (m/s) |  |  |
| hf along the headrace (m) |  |  |
| Vjet (m/s) |  |  |
| P (MW per metre of width) |  |  |
| η0 (m) |  |  |
| k (s2/m) |  |  |
| ηmax (m) |  |  |

# Answers (for the tutor)

Computed from the data in “The scheme”. Two or three significant figures are all the sheet supports. The app column is one run of the scene at Ds = 3.0 m, settled 60 s, read over a 20 s mean and then slammed. λ = 0.03 is the app's own delivered friction: a least-squares fit of the headrace HGL over x = 24–44 m (clear of the entry recovery) on a 60 s mean gives a slope of 1.61 × 10−3 at V0 = 2.51 m/s, which is λ = 0.030.

|  | Part 1 — Hydropower | predicted | app |
| --- | --- | --- | --- |
| (a) | V0 = q0/Dh = 7.6/3.05 | 2.49 m/s | 2.51 |
| (b) | hf = λ·(L/2Dh)·V02/2g = 0.03 × (42.4/6.10) × 0.316 | 0.066 m | 0.068 (λ = 0.030) |
| (c) | Vjet = √(2g·(H − hf)), H = 21.9 m, hf = 0.066 + 0.064 = 0.130 m | 20.7 m/s | 20.6 |
| (d) | P = ρ·g·q0·(H − hf) = ½·ρ·q0·Vjet2 | 1.62 MW/m | 1.62 |
| (e) | P is greatest at hf = H/3; since hf ∝ q2, q* = q0·√(H/3hf) = 7.6 × 7.50 | 57 m2/s | — |
|  | Part 2 — Unsteady flow | predicted | app |
| (a) | η0 = −(V02/2g + hf) = −(0.316 + 0.066) | −0.38 m | −0.38 ± 0.02 |
| (b) | k = −η0/V02 | 0.0616 s2/m | 0.0602 |
| (c) | Y = L·Dh/(2g·k·Ds) | 35.7 m |  |
| (d) | ηmax = Y·[1 − exp((η0 − ηmax)/Y)] | 4.97 m | 4.95 |

The app reproduces the calculated crest to 0.4% (4.95 m against 4.97 m). η0 agrees to 0.5% when read off the free surface; read off p/ρg part-way down the shaft it comes out ~0.07 m high and twice as scattered, which is a property of the reading, not of the friction. Other widths, when the class ran one per digit: 2.5 m → 5.47 (app 5.22), 3.5 m → 4.58 (app 4.62), 4.0 m → 4.27 (app 4.23), 4.5 m → 4.01 (app 4.03), 5.0 m → 3.80 (app 3.89), 5.5 m → 3.61 (app 3.75), 6.0 m → 3.44 (app 3.55), 6.5 m → 3.30 (app 3.25), 7.0 m → 3.17 (app 3.31).